Guide Specification

Wally Bench

  - GENERAL

  - WORK INCLUDED

    - Provision of steel and wood bench

  - RELATED WORK

    - Section 033000 Cast-in-Place concrete

    - Section 061000 Rough Carpentry

    - Section 062000 Finish Carpentry

    - Section 129300 Site Furnishings

  - SUBMITTALS

    - Product Data: Manufacturer's standard catalog cut sheets.

    - Samples: As required for color selection or material thickness only.

    - Shop Drawings: For custom applications, showing critical sizes and dimensions for installation and integration with other work.

    - Environmental Product Declarations (EPDs): Submit third-party verified and product-specific EPDs for the specified materials, demonstrating compliance with ISO 21930, 14025, and 14044, or equivalent standards.

  - DELIVERY, STORAGE AND HANDLING

    - Unwrap & inspect benches after delivery for signs of damage during transit.

    - Protect benches from damage during storage and handling.

    - Store benches indoors if possible. Do not stack.

  - PROJECT CONDITIONS

    - Contractor to provide adequate structure to support benches and its users.

    - Protect units from damage by adjacent work.

1.6 WARRANTY

- Product will be free from defects in material and/or workmanship for a period of 3 years from invoice date.

- Warranty does not apply to damages from alteration, misuse, or installation damage.

- Normal use of these products may result in scratches, nicks, and dents. These are considered normal wear and tear, and are not the responsibility of the manufacturer.

- Tournesol Siteworks will, at its option repair, replace, or refund the purchase     price of products that are deemed defective by an authorized Tournesol Siteworks representative.

  - PRODUCTS

  - ACCEPTABLE PRODUCTS/MANUFACTURERS

Wally Bench, manufactured by Tournesol Siteworks LLC. 2930 Faber St., Union City, CA 94587  Tel: (800) 542-2282  Tournesol.com

  - FLAT WALLY BENCH – TYPE A

    - Materials

      - Lumber shall be Thermally Modified Oak

Finished dimensions:

Rectangular planks 1” x 3-1/2” with 1/8” radius edges and corners.

    - Construction

      - Lumber – Double stacked front boards with hidden fasteners. Qty (5) seating surface boards with 1/8” spacing. Profiled and/or shaped with minimum surface smoothness of 20 KCPI. No tear-outs or knife-knicks. Pilot holes required for all attachment points.

      - Powder-coated carbon steel cantilever support arm weldments – Laser cut, machined, and fully welded.

      - Powder-coated carbon steel board straps – Laser cut and machined.

      - All hardware to be internal, hidden and not visible from the top of the bench.

    - Performance characteristics

      - Lumber – All corners and edges to be rounded or eased. All attachment points to be internal and not visible from the top of the bench.

      - Powder-coated carbon steel cantilever support arm weldments – All exposed sharp edges and weld splatter removed.

      - Powder-coated carbon steel board straps – All exposed sharp edges removed.

    - Finish: specified finish; factory finished.

      - Carbon steel –

Following fabrication, the bench supports shall be cleaned and treated with an iron phosphate process prior to the coating application. This process shall include a non-chromated alkaline cleaner, and an iron phosphate treatment, followed with an acidic sealer for maximum adhesion. Corrosion-resistant zinc undercoat shall be.

applied, 1-2mils thick. The protective powder coat shall be polyester or polyester TGIC powder, minimum 4 mils thick. The following application parts shall be baked until properly cured.

    - Sizes: Refer to catalog for standard sizes. Custom sizes as per approved shop drawings.

  - CASCADE WALLY BENCH – TYPE C

    - Materials

      - Lumber shall be Western Red Cedar, Douglas Fir, Ipe, or Recycled Plastic Lumber

        - Finished dimensions:

Square 3-1/2” x 3-1/2’ with 1/4” radius edges and corners.

Rectangular 3-1/2”tall x 1-1/2”wide with 1/4” radius edges and corners

      - Powder-coated carbon steel cantilever support arm weldments – 3/8”thk ASTM A36 hot rolled plate & 2-1/2”x2-1/2”x11guage wall A500 square steel tube.

      - Powder-coated carbon steel board straps – 3/8”thk ASTM A36 hot rolled steel.

      - Hardware – Stainless steel grade 18-8 wood screws

    - Construction

      - Lumber – Qty (2) Square boards. Qty (6) rectangular with 1/2” spacing. Profiled and/or shaped with minimum surface smoothness of 20 KCPI. No tear-outs or knife- knicks. Pilot holes required for all attachment points.

      - Powder-coated carbon steel cantilever support arm weldments – Laser cut, machined, and fully welded.

      - Powder-coated carbon steel board straps – Laser cut and machined.

      - All hardware to be internal, hidden and not visible from top of bench

    - Performance characteristics

      - Lumber – All corners and edges to be rounded or eased. All attachment points to be internal and not visible from top of bench.

      - Powder-coated carbon steel cantilever support arm weldments – All exposed sharp edges and weld splatter removed.

      - Powder-coated carbon steel board straps – All exposed sharp edges removed.

    - Finish: specified finish; factory finished.

      - Carbon steel –

a.: Following fabrication the bench supports shall be cleaned and treated with an iron phosphate process prior to the coating application. This process shall include a non-chromated alkaline cleaner, and an iron phosphate treatment, followed with an acidic sealer for maximum adhesion. Corrosion-resistant zinc undercoat shall be applied, 1-2mils thick. Protective powder coat shall be polyester or polyester TGIC powder, minimum 4 mils thick. Following application parts shall be baked until properly cured.

    - Sizes: Refer to catalog for standard sizes. Custom sizes as per approved shop drawings.

# EXECUTION

    - Planter mounted

      - Use hardware and backer plate provided in planter mount kit provided by manufacturer. Kit is not standard, must be added to bench at time of order.

    - Wall mounted

      - Ensure wall is structurally sound and engineered to hold weight of bench and occupants. Wall at bench mounting points to be flat and coplanar.

      - Source 3/8” diameter non-corrosive anchoring hardware approved for use in the wall material the bench is to be mounted on. Follow anchor manufacturers recommendations for installing anchors.

# PREPARATION

    - Prior to planter fabrication, the contractor shall verify as-built dimensions of area to ensure proper size, fit and quantity required.1

# INSTALLATION

    - Dry fit bench onto the mounting surface to ensure fit-up before locating anchors.

    - Ensure product is level and spacing between units is as specific